| WRC C3 Lighting Control System Guide Power Supply： 1 、DC 6-7.4V, Build-in BEC with ESC power supply. Lights Control Details： |  |  |
| --- | --- | --- |
| 1、White Daytime Running Lamp(DRL) on when power on ，Red Stop Lamp on when braking，White backup lamp on when reversing，Orange turning lamp when turning. 2、Lighting Control By Hand-wheel： (Head Lights ) ：Turn hand wheel quickly forward (right) full stroke and then back, twice, wait 3 seconds (Without other operations of RC).The headlights are on, Repeat operations to turn off. When headlights on, the brake lights are on weak; The brake lights are strong when braking. Turn off the headlights, the brake lights are off. (Double Flash Lights ) ：Turn hand wheel quickly back (left) full stroke and then back, twice,wait 3 seconds (no other operations of RC).The double flash lights are on, DRL turn into orange,tail lights are flashed.Repeat operations to turn off. DRL turn back white. (DRL Color ) ：Turn hand wheel quickly forward (right) full stroke and then back, 3 times, wait 3 seconds (Without other operations of RC).The DRL turn into orange, Repeat operations to turn back. The changes are valid When double flash lights are on. (DIY Light) ：Turn hand wheel quickly back (left) full stroke and then back, 3 times, wait 3 seconds (Without other operations of RC).The DRL turn into orange, Repeat operations to turn off. Warning：Pay attention to positive and negative terminals and make sure of the DC 6-7.4V when DIY to avoid the damage of lighting system！ 3、If the turn signal lights are opposite to the hand wheel, press the reverse button on the light control board once. Then All above ways of turning on the lights by the hand wheel have to be reversed. 4、If the reverse lights are on when moving forward, quickly press the reverse switch on the light control board twice. |  |  |
| C3 Light Group Parts List |  |  |
| - + C3Linght Control Boad×1 | 4P IN Signal Cable ×1 S S + - + - CH CH + 1 2 - S S + + - - |  |
|  | CH1 CH2 + - |  |
|  | 4P IN Extra Signal Cable ×1 |  |
| Left Taillight Set×1 Right Taillight Set×1 |  | DRL Set×2 Headlight Set×1 |

Connecting Constructions：

to ESC

CH2 Tag

S

+

-

CH2

To RecevierCH2

1、Following the diagram to connect the signal cables to the receiver CH1 and CH2, and 4P IN plugs to the matched 4P IN sockets；

2、Connect lights set to the ports in the light control board respectively；

3、Connect signal cables to the 4P in extra cable before installing the body shell.

Elementary Diagram：

CH1Tag

To ReceiverCH1

S

+

-

S

+

-

To Steering Motor                                 4P IN Plug With Lock

C H 1

+ - S + -

Signal Cable

To Light Control Board Signal Receiving 4P IN Plug

4P IN Plug With Lock

C H 1

CH2

+

-

Signal Converting Cable

4P IN

4P IN

Signal Cable Port

Right Taillight

Left Taillight

4P IN

4P IN

Right Taillight Set

Left Taillight Set

Reversing Button

C3 Light Control Board

6-7 .4V Output Terminal(Operated By RC)

FUTABA/JR Terminals

are Availablewith DC 6-7.4V

+ -

LED Stripe for DIY refiting

Connect the positive and negative accurately！

Right DRL Port

Right DRL Port

3P IN

3P IN

Headlight Port

2P IN

Headlight

White

10mm LED

DRL

Orange & White         2×3×4mm Square LED

Headlight

White

10mm LED

DRL

Orange & White         2×3×4mm Square LED